AMSecBench

AI-Native Security for Additive Manufacturing: Can Large Language and Vision-Language Models Detect Toolpath Sabotage — and Can They Defeat Design Obfuscation?

Research Proposal

Harsh Vardhan Gupta (proposer)  ·  Prof. Nikhil Gupta, FASM  ·  Prof. Ramesh Karri
NYU Tandon School of Engineering / NYU Center for Cybersecurity

August 2026  ·  Code: github.com/Harshelite2503/amsecbench

# Abstract

Additive manufacturing (AM) is a cyber-physical process: a design file becomes a toolpath (G-code) becomes a part, and every hop is a place where an attacker can degrade the part without changing how it looks. The AM-security literature — including the Proc. IEEE survey by Gupta, Karri and colleagues — established the attack taxonomy (internal voids, infill reduction, parameter manipulation, dimensional sabotage) and two defensive families: verification of the toolpath against the design, and design obfuscation so a stolen file prints the wrong part. Large language and vision-language models (LLMs/VLMs) have not been evaluated on either side of this problem. We propose AMSecBench, a fully synthetic, labelled benchmark with two tracks. Track A (defense) asks whether LLMs/VLMs can detect G-code sabotage, with and without a trusted reference, better than rule-based and classical-ML detectors. Track B (red team) asks whether LLMs/VLMs can defeat the design-based obfuscation schemes developed in the collaborating group by recovering the intended design from an obfuscated file. A working benchmark exists: 141 G-code files (45 clean incl. benign parameter variants, 96 attacked across 7 attack classes on 5 parts). Baselines already show the central difficulty: a reference-comparison detector catches 75–100% of attacks but falsely flags 33% of legitimate parameter variants, and reference-free ML reaches AUC 0.86 with a 38% false-positive rate. Outcomes are a benchmark paper at a security venue, a defense/attack findings paper, and a proposal-ready basis for NSF SaTC / DoD funding.

# 1. Problem Statement

## 1.1 The AM pipeline is attackable at every hop and the damage is invisible

A 3D-printed part is produced from a CAD/STL file through slicer software into G-code executed by the printer. Prior work (dr0wned, 2017; Gupta et al., 2020; Rossel et al., USENIX Security 2025) has shown that small edits to the G-code — removing a few millimetres of extrusion to create a void, reducing infill in a band of layers, lowering nozzle temperature for a few layers — yield parts that pass visual inspection and fail in service. Distributed and outsourced printing widens the attack surface: the file leaves the designer’s control.

## 1.2 Existing detectors are brittle

Verification approaches compare the toolpath to the design or to a trusted reference. They work when the reference and the suspect were produced with identical settings, but legitimate operators change infill, layer height, temperature and speed routinely; a naive comparison flags those changes as attacks. Reference-free anomaly detectors must instead generalise across geometries they have never seen. Neither approach reasons about intent — whether a change is a plausible operator choice or a degradation.

## 1.3 Obfuscation defenses have never faced an AI adversary

Design-based security (Chen & Gupta; NYU) embeds decoy features and parameter-gated ‘key’ features so a stolen STL prints a wrong or failed part unless the process recipe is known. These schemes were evaluated against human reverse engineers and conventional tooling. Multimodal LLMs can now read STL geometry, renders and G-code, and reason about mechanical function; whether they can identify and strip decoys is unknown — and directly determines whether these defenses remain valid.

## 1.4 LLMs are entering manufacturing workflows without a security evaluation

LLM assistants are being integrated into slicers, print-farm management and CAD tools. There is no benchmark that measures their value as a detector or their threat as an attacker in AM. Both the defensive community and the roadmap literature (Advanced Intelligent Systems 2026) identify this as an open problem.

# 2. Objectives

O1 — Benchmark: an open, synthetic, reproducible dataset of clean, benign-variant and sabotaged G-code with ground-truth labels and attack metadata, plus rendered toolpath images.

O2 — Defense evaluation: measure LLM (text) and VLM (image) detection accuracy, false-positive rate on benign variants, attack-class attribution and calibration, with and without a trusted reference, against rule and ML baselines.

O3 — Red-team evaluation: implement the group’s obfuscation schemes on benchmark parts and measure whether LLMs/VLMs recover the intended design (IoU) and identify parameter-gated features.

O4 — Hardening: use red-team results to propose obfuscation variants that resist multimodal AI, and LLM-in-the-loop detection that reasons about operator intent.

O5 — Real-slicer validation: reproduce the key results on PrusaSlicer/Cura output and, where available, on printed specimens from the collaborating lab.

# 3. Background and Related Work

AM security surveys: Gupta, Karri et al. (Proc. IEEE 2020) taxonomise threats and countermeasures including obfuscation and embedded security features. Attack studies: dr0wned (Belikovetsky et al. 2017) demonstrated void-sabotage of a drone propeller; Rossel et al. (USENIX Security 2025) analysed malicious G-code at scale across slicers. Detection: probabilistic anomaly models on printer telemetry (Yoginath et al. 2022), power side-channels (PowerGuard 2024), design-vs-toolpath verification (Mahmood et al. 2025), hardware firewalls (Firewall3D 2026). Design-based security: Chen (PhD, NYU 2019, advised by N. Gupta) — embedded tracking codes and parameter-gated features. AI in AM: roadmap papers call for LLM/VLM quality assurance but no security benchmark exists.

# 4. Approach

## 4.1 Track A — sabotage detection benchmark

| Component | Design | Status |
| --- | --- | --- |
| Parts | 5 parametric parts: block, bracket with holes, thin-wall tube, ASTM-style dog-bone, grid plate (generated, no downloads) | Done |
| Slicer | Minimal inspectable FDM slicer (perimeters + rectilinear infill, absolute E) so every attack has a known effect | Done |
| Attacks | infill reduction, void insertion (segment-split), layer-height change, mid-print temperature drop, 3% XY scaling, under-extrusion band, infill toolpath jitter | Done |
| Benign variants | 8 legitimate parameter changes per part (infill 0.2/0.4, layer 0.15/0.25, temp 200/220, speed 30/60) — detectors must not flag these | Done |
| Features / renders | 17 per-file features (layer statistics, extrusion-rate, infill ratio, temperature, bbox, jitter, travel-in-fill); PNG toolpath renders | Done |
| Baselines | Rule detector (reference comparison); logistic / random-forest / gradient-boosting, leave-one-part-out | Done |
| LLM / VLM detectors | Claude on per-layer summaries with reference; on raw G-code without reference; on rendered images | Code ready; runs pending |

## 4.2 Track B — obfuscation red team

Decoy features: extra bosses added to the intended design; the attacker must decide which features to remove.

Parameter-gated keys: 0.25 mm ribs that only print with a specific nozzle profile and carry a load path.

Adversary: LLM/VLM given STL statistics, multi-view renders and (optionally) the G-code, asked to output the intended design as a feature list / edit script; scored by volumetric IoU against ground truth and by whether keys are identified.

Threat model: attacker has the file but not the process recipe; no physical access; this mirrors the design-theft scenario the defenses target.

## 4.3 Ethics and scope

All artefacts are synthetic and generated locally. No firmware, network or physical printer is targeted, and no exploit tooling against real systems is produced. Track B evaluates academic defenses published by the collaborating group, with their authors, which is standard practice for security research.

# 5. Preliminary Results

The Track A benchmark is built and two baseline families have been evaluated on 141 G-code files (45 clean/benign, 96 attacked; attacks that had no geometric effect, e.g. a void placed in a part with no infill, are excluded rather than mislabelled).

Figure 1. Detection rate per attack class for the baseline detectors; the last column is the false-positive rate on benign parameter variants.

## 5.1 Reference comparison is sensitive but not specific

The rule detector, which compares a suspect file with the trusted reference, detects infill reduction 100%, void insertion 75%, layer height change 100%, temperature shift 100%, scaling 100%, under extrusion 100%, toolpath jitter 100%. However it flags 33% of legitimate parameter variants as attacks — every layer-height and temperature change an operator might legitimately make is indistinguishable from sabotage without reasoning about intent. This is the specificity problem that motivates an LLM-in-the-loop detector.

## 5.2 Reference-free ML generalises only partially to unseen parts

| Model | AUC | F1 | False-positive rate |
| --- | --- | --- | --- |
| logreg | 0.667 | 0.57 | 0.38 |
| random_forest | 0.857 | 0.881 | 0.38 |
| gbc | 0.825 | 0.913 | 0.22 |

Under leave-one-part-out evaluation (the model never sees the test geometry), the best model reaches AUC 0.86 but still mislabels 38% of benign files. Void insertion and infill reduction are the hardest classes because their signature is local (a few layers) while the features are global. These are exactly the cases where a model that can read the per-layer table and reason about where the change occurred should help.

## 5.3 Engineering findings

A void attack must split toolpath segments at the void boundary; naïve endpoint deletion is a no-op on edge-to-edge infill lines — a subtlety that likely affects earlier synthetic studies.

Per-layer features are required to see small voids; whole-file ratios dilute them below the benign-variation noise floor (benign variants move the travel-in-fill feature by ≤0.024, real voids by 0.05–0.32).

Thin-wall parts carry no infill, so void/infill attacks are inapplicable — the benchmark records this rather than mislabelling.

Figure 2. Rendered toolpath of the bracket with an inserted void (right: the affected layer).

# 6. Work Plan

| Phase | Weeks | Activities | Deliverable |
| --- | --- | --- | --- |
| 1. LLM/VLM detection | 1–3 | Run Claude detectors (summary+reference, raw no-reference, image); calibration; attribution accuracy; cost | Track A results |
| 2. Real-slicer port | 3–5 | Regenerate attacks on PrusaSlicer/Cura G-code; confirm findings transfer | Benchmark v1.0 |
| 3. Obfuscation red team | 4–8 | Implement Chen–Gupta schemes on benchmark parts; LLM/VLM de-obfuscation; IoU scoring; hardening variants | Track B results |
| 4. Physical validation | 6–10 | Print a subset (clean vs sabotaged) in the Gupta lab; µCT/mechanical test to confirm attack effect | Validation data |
| 5. Writing / proposal | 8–12 | Benchmark + findings papers; NSF SaTC / DoD white paper | Submissions |

# 7. Expected Outputs

| Output | Venue options | Contribution |
| --- | --- | --- |
| Benchmark + detection paper | USENIX Security / IEEE S&P (dataset & benchmark track); ACSAC; Additive Manufacturing | First LLM/VLM evaluation for AM sabotage detection with benign-variant false-positive analysis |
| Red-team paper | IEEE S&P / CCS workshops (CPS-SPC); Journal of Manufacturing Systems | Whether design obfuscation survives multimodal AI; hardened variants |
| Open benchmark + code | GitHub + Zenodo | Reusable, extendable to other AM processes |
| Funding proposal | NSF SaTC, DoD/ARL manufacturing security, NYU CCS seed | Both PIs have track record; benchmark provides preliminary data |

# 8. Resources and Budget

| Item | Estimate |
| --- | --- |
| LLM/VLM detection runs (Track A, ~150 files × 3 modes) | ≈ US$50–100 |
| Red-team runs (Track B, 10 parts × 2 schemes × repeats) | ≈ US$30–60 |
| Printing + testing of validation specimens | Gupta lab consumables; µCT time |
| Compute | Laptop-scale |

# 9. Roles

Harsh Vardhan Gupta: benchmark engineering, attack/detector implementation, LLM/VLM experiments, analysis, drafting.

Prof. Nikhil Gupta: AM process and materials expertise, obfuscation schemes, physical validation, co-authorship.

Prof. Ramesh Karri: threat modelling, security-venue framing, red-team methodology, co-authorship and funding strategy.

# 10. Risks and Mitigations

| Risk | Mitigation |
| --- | --- |
| Synthetic slicer differs from real slicers | Phase 2 port to PrusaSlicer/Cura; attacks operate on G-code so they transfer |
| LLM cost / context limits on raw G-code | Per-layer summary mode; chunked windows; Message Batches |
| Dual-use concern in Track B | Synthetic parts only; academic defenses evaluated with their authors; no real-system exploitation; responsible disclosure norms |
| Small part library | Parametric generator scales to hundreds of parts; add open-source functional parts |

# References

Gupta N., Karri R. et al. A survey of cybersecurity of digital manufacturing. Proceedings of the IEEE, 2020.

Chen F. Design-based security strategies for the additive manufacturing cyber-physical systems. PhD thesis, NYU, 2019.

Belikovetsky S. et al. dr0wned — cyber-physical attack with additive manufacturing. USENIX WOOT, 2017.

Rossel J. et al. Security implications of malicious G-codes in 3D printing. USENIX Security, 2025.

Yoginath S. et al. Stealthy cyber anomaly detection on large noisy multi-material 3D printer datasets. ACM AMSec, 2022.

Mahmood M.A. et al. A novel framework for identification of cyber-physical attacks in additive manufacturing. Progress in Additive Manufacturing, 2025.

Asgar S.A.G., Reddy N. Firewall3D: a hardware firewall for defending 3D printers against firmware attacks. arXiv, 2026.

Zolfagharian A. et al. Roadmap on artificial intelligence-augmented additive manufacturing. Advanced Intelligent Systems, 2026.

Gupta N., Beckwith C. Architected Metamaterials: Design Principles and Properties. Springer, 2025.